      Quick Installation Guide            [image: image1.png]



ZTE ZXDSL 852 ADSL Modem

Device Introduction

[image: image2.jpg]



Figure 1

1 POWER indicator
2 DATA indicator

3 LINK indicator
4 USB port
5 LINE port
Wiring Diagram

Note:

Do not connect the modem with your PC until install the driver properly.

Please wait for at least 5 seconds before insert and remove USB cable.
[image: image3.png](U]

.




 
Figure 2
Driver Installation

Just plug the modem on a USB port of the PC, the Plug & Play module will guide you through the installation procedure.
Note : Before beginning the software setup, insert the installation CD into the C D drive.
1. Run the install file “setup.exe” in CD, a dialog box will appear as shown below.
[image: image4.jpg]Instaling the ADSL Modem divers.
Please wai.




2. A dialog box will prompt you to connect the MODEM, as shown below.
 [image: image5.jpg]Yo need now 1o pluginyour ADSL modem n order to
complete the insallaion

Carcel





3. A dialog box shows the installation is complete as shown below. Click the “OK” button.
[image: image6.jpg]Vour madem nstalation is successful,

o





4. A new icon [image: image7.jpg]


 will appear in the taskbar on the lower right corner of the screen after installation.
[image: image8.jpg]



For more information please consult the User’s Manual on the CD-ROM.

Configuring the Computer / Notebook

ZTE ZXDSL 852 ADSL Modem comes preconfigured for SLT Broadband. Customer will have to enter the username and password only. 

· Double click on the following Icon in the desktop.

                     [image: image9.png]



· Enter the username and password in the Connect ZTE ZXDSL 852 window.
    [image: image10.png]User ame:

Password

[ —T S ——

Me only
Aryane who ses his computer

| | )





However, if you encounter any problem please follow the steps below. 

1. Click Start button on the task bar and select Settings > Control Panel
2. Double click on System icon.
    [image: image11.jpg]B Control Panel

Ble et

Qs -

dress B contrl panel

vew

@ swikchto Category Vew

R Windows U
@) Heb and support

Favortes

Tools  tielp

(| O search [ polders | [~

Viard | Madem

@ =

Regonaland  Scanners and

19
Sordsand  oneh
it

P £

Wiireless
Network set.

502 nformation about your computer system, and change settings for hardware, performance, and automatic updates





3. Select Hardware > Device Manager > Network Adapters
        [image: image12.jpg]System Properties

System Restore Automatic Updates Remate.

General Computer Name Hardware Advanced

Device Manager

The Device Manager lsts all the hardare devices nstaled
on your compuar Lse the Device Manager ta change the
propeties of any device.

Diivers
@ Diiver Sigring lts you make surethat installed divers ae

compalble with Windows. Windows Update lts you set up
how Windows cannects to Windows Lipdete for divers.

Hadware Profles

Hardwate profes provide a way for you to et up and store
diferent hardware corfiguratons.

Hardware Frofies





            [image: image13.jpg]= B G

8 0SL vadem

3 Computer

< Dikdives

2 Disply adepters

2 DVDJCD-ROM drives:

2 Flogpy disk contolers

T3 Foppy dis ives

2 IDE ATAJATAPI controlers

S Keyboards
Vice and other pinting devices
Woniors

B Network adapters
B Realtek RTL8139/810x Family Fast Ethernet NIC





4. In the WAN / ATM / ADSL miniport Properties window, select PPP tab and enter the following parameters. 

· VCI Identification

VPI – 8

VCI – 35

· PPP Encapsulation

PPP over Ethernet Bridged LLC/SNAP (RFC 2516 / RFC 1483)

Disable Proxy Settings:

· Windows Desktop > right click Internet Explorer (IE) > click Properties
· “Internet Properties” window > click Connections tab > select Never dial a connection > click LAN Settings…

· “Local Area Network (LAN) Settings” window > remove the tick for the option for the Automatic configuration > remove the tick for option box for Proxy server z. click OK > click OK again to exit from “Internet Properties” window. 
Once the PC / Notebook configuration and Modem installation is completed you may proceed to surf the internet.
Uninstallation

1. Click on [image: image14.png]R Start |



, select ALL Programs(ZTE ZXDSL 852, then click Uninstall Drivers as shown below.

 [image: image15.jpg]) Internet
B2 Internet Explorer

®

et Program Access and Defauls

% windows Catalag
) E-mail ®

L& cutiook express Windows Update

@ Accossores ,
o @ canes ,
s ,
B) vindorsodsio] D =0
Intermet Exlorer

3

() Outlock Evpress
@) ror vt

L2 Remote Assistance 5
® vindows Media Plyer
Fies and Settings T
Y5 wizard 48 windows Messenger
) portom | b Wi Hovi ke
yperTerminal
e ank - PrsonaEdn ,
WiRAR »

L zose2





2. A dialog box will appear, you need to click “Yes” to confirm deletion as shown below
        [image: image16.jpg]Do you eally want o uninstal e ADSL divver and
femave allthe relted fles fram the system 7

Vor ] No |





3. A dialog box will prompt you to unplug the modem as below
     [image: image17.jpg]L\ P ezt theUss ol o s AL s e

o





4. A dialog box will prompt you to restart Windows as shown below
  [image: image18.jpg]Vol need o restar the Operating System n ordet (o cormpite the urinstaltion.
Some fbrayfles st loaded wil be remaved ding this process.

Do you want o restat now

V] No





· If still you have any difficulty call 1243 and SLT ADSL helpdesk will assist you to complete the installation process.
· Visit SLTnet Broadband in www.sltnet.lk for more information. 
(1) Computer 


(2) Phone 


(3) USB Cable


(4) RJ-11 Tel Cable 	


(5) Splitter 


(6) RJ-11 Tel Cable


(7) RJ-11 Tel Cable 	


(8) Phone Jack 


(9) ZXDSL 852








1
2
3

